In the Mastermind game, a player tries to discover a secret code by trial and error.

In our implementation, the secret code consists of 4 digits. Each digit is in the range [0-3].

Write the Mastermind subroutine in ARM assembly language. You must follow this prototype:

int Mastermind(int guess[4], int secret[4], int usedGuess [4], int usedSecret [4]);

The four parameters are addresses of arrays containing four integers.

In the Reset_Handler, before calling the subroutine, you can initialize the elements of guess (the player’s try) and secret (the code to be discovered) with values of your choice. Instead, all the elements of usedGuess and usedSecret must be initialized to zero.

The Mastermind subroutine computes:

- the exact matches: the number of digits that have the same value and appear in the same position in both arrays (guess and secret);

- the partial matches: the number of digits in guess that are not exact matches, but appear in a different position in secret.

Example: guess = [0, 1, 2, 3]; secret = [1, 2, 2, 0].

There is one exact match (digit 2 in the third position) and two partial matches (digits 0 and 1).

The subroutine implements the following pseudocode:

exact = 0;

partial = 0;

for (i = 0; i < 4; i ++) {

if (guess[i] == secret[i]) {

exact ++;

usedGuess[i] = 1;	/* mark the digit as used */

usedSecret[i] = 1;

}

}

for (i = 0; i < 4; i ++)

for (j = 0; j < 4; j ++)

if (usedGuess[i] == 0 && usedSecret[j] == 0 &&

guess[i] == secret[j]) {

partial ++;

usedGuess[i] = 1;

usedSecret[j] = 1;

break;	/* ends the inner loop */

}

return (2exact -1) << 4 + (2partial -1)

In this pseudocode, the arrays usedGuess and usedSecret are used to mark digits that have already been matched. A value of 0 indicates that the digit has not been matched yet, whereas a value of 1 indicates that it has already been matched.

In the example above, at the end of the procedure we have:

usedGuess = [1, 1, 1, 0]

usedSecret = [1, 0, 1, 1]

exact = 1

partial = 2

(2exact – 1) = 1

(2exact – 1) << 4 = 16

(2partial – 1) = 3

The subroutine returns 19.